Plant Watering Robot — Hardware Purchase List (v2, sourced + treatment-aware)

Sourced June 2026  ·  Phase 1: single-zone moisture-based watering  ·  hose/pipe + solenoid-valve variant

Builds on Hardware_Purchase_List_sourced_Jun2026.docx — adds one relay upgrade (item 5) informed by the planned treatment expansion. Phase 1 scope is unchanged: moisture check + watering only. Original documents untouched.

# Legend & changes from previous sourced list

🟢  Green row — upgraded component: buy this version instead of the original

🟡  Amber row — action required before ordering

Plain rows — unchanged from previous sourced list

- Item 5 (relay module) — UPGRADED from 1-channel to 4-channel. Cost difference is negligible (~₹60–100 extra). Phase 1 uses only channel 1 for the solenoid valve; the remaining 3 channels are ready for future treatment pumps (pH-up, pH-down, nutrient) without buying a second relay board. This is the only Phase 1 BOM change warranted by the planned treatment expansion — everything else (MCP3008's 8 ADC channels, Pi GPIO capacity) is already sufficient.

- Items 6 & 7 (solenoid valve + fittings) — amber warning carried over from previous list: verify NPT vs BSP thread standard on item 6 before buying item 7.

- Item 1 (Pi Zero 2 W kit) — Amazon.in link carried over. If it has gone OOS, fall back to Pi 4 Model B 1GB.

- Items 2, 3, 4, 8–11 — unchanged.

# Bill of materials

| Item | Exact spec / notes | Where to buy (sourced June 2026) | Approx. price |
| --- | --- | --- | --- |
| 1. Raspberry Pi Zero 2 W — Basic Kit | Kit includes: Pi Zero 2 W board (quad-core 64-bit, WiFi + BT), aluminium heatsink, USB OTG cable, Mini HDMI adapter, 40-pin GPIO header. ⚠ Header likely un-soldered — solder on, or buy a press-fit solderless header separately (~₹50–100). | amazon.in — iRasptek Pi Zero 2W Kit (heatsink, OTG, HDMI adapter, 40-pin header) Fallback: Pi 4 Model B 1GB at Silverline/Robocraze (~₹11,000) if kit goes OOS | ~₹5,000 (kit + accessories) |
| 2. MicroSD card, 16GB+ Class 10 / A1 | e.g. SanDisk Ultra — Pi OS + application storage | amazon.in — search "16GB microSD class 10" | ₹300–500 |
| 3. Capacitive soil moisture sensor | v1.2 or v2.0, CAPACITIVE (not resistive — corrodes), analog output, 5V | robokits.co.in — Capacitive Soil Moisture Sensor V1.2 (₹83, confirmed June 2026) (also: robu.in — soil sensor category) | ₹83–250 |
| 4. MCP3008 — 8-channel 10-bit ADC | SPI interface, DIP-16 package — Pi has no analog input pins, this bridges the moisture sensor. 8 channels gives headroom for future pH + EC sensors without replacing this chip. | robu.in — MCP3008 DIP-16 (also: dnatechindia.com — MCP3008) | ₹250–650 |
| 5. 4-channel 5V relay module ↑ upgraded from 1-channel | Opto-isolated, 5V, 4 channels — Phase 1 uses only channel 1 (solenoid valve). Remaining 3 channels are spare headroom for future treatment pumps (pH-up, pH-down, nutrient) without buying a second relay board. Only ~₹60–100 more than the 1-channel equivalent — worth buying now. | ✓ UPGRADE from 1-channel — same price range, 4× capacity robocraze.com — 4-Channel 5V Relay Board with Optocoupler (₹144) (also: robu.in — 4-Channel Isolated 5V 10A Relay w/ Optocoupler) | ₹100–200 (vs ₹60–150 for 1-ch — negligible delta) |
| 6. 12V DC solenoid valve, ½" — normally closed (NC) | Gates the always-on hose/pipe line — NC means it fails SHUT on power loss. ⚠ Robu.in listing (SKU 31396) describes outlets as NPT, not BSP. Verify thread standard before buying fitting (item 7) — NPT and BSP are NOT interchangeable under line pressure. | ⚠ VERIFY THREAD STANDARD (NPT vs BSP) BEFORE ORDERING ITEM 7 robu.in — 12V DC Solenoid Water Valve ½" NC (SKU 31396) (also: robocraze.com — 12V Solenoid Valve ½" NC) | ₹350–750 |
| 7. Hose/pipe fittings | ½" hose-to-valve adapter + thread tape. ⚠ Buy AFTER confirming item 6's actual thread standard (NPT or BSP) — ask your plumbing shop for a matching adapter. | Local plumbing/hardware shop — confirm thread standard from item 6 first | ₹50–150 |
| 8. Outlet tubing | Food-grade silicone or reinforced PVC, ID ~8–12mm (matched to valve outlet), ~1m | robu.in — silicone/PVC tubing search (also: local hardware shop) | ₹100–200 |
| 9. Separate 12V/1A power adapter (for the valve) | Generic 12V/1A adapter — do NOT power the valve from the Pi's 5V rail | Local electronics shop / amazon.in (generic) | ₹150–300 |
| 10. MB102 breadboard + jumper wire kit | 830-point breadboard + 140pc jumper wires (M-M, M-F, F-F) | robu.in — MB102 Breadboard + 140 Jumper Wires Kit | ₹250–400 |
| 11. 1N4007 diode (pack of ~10–30) | 1A 1KV standard recovery diode — flyback protection across the solenoid valve coil | robu.in — 1N4007 1W Diode (Pack of 30) | ₹20–50 |

# Before placing the order

- Confirm Pi Zero 2 W kit (ASIN B0BRWNQLCW) ships to your pin code and 40-pin header is included. Solder it on (or buy a press-fit header ~₹50–100).

- Open the solenoid valve listing (item 6, SKU 31396) and confirm thread standard (NPT or BSP). Tell your local plumbing shop which it is when buying the fitting (item 7).

- Buy the 4-channel relay board (item 5) — not the 1-channel version. Use only channel 1 for Phase 1; label or tape off the other 3 channels so they're clearly unused until treatment expansion.

- Treatment expansion hardware (peristaltic pumps, pH sensor, EC sensor, nutrient reservoirs) is NOT in this list — those belong to a future phase. See memory/plant-treatment-expansion.md for the full plan.